LQR control

- Modern method of control

- Optimal control technique –

- uses Riccati equation for finding optimum feedback gain K

- K is found such a that Performance Index J is minimized

- 2 matrices – Q  and R are selected by designer as per the specific requirement

1] Satellite control system –

Here, the need is to reduce the energy consumption as the system operates on Solar Power. In this case, we can keep high value for R and penalize the actuator efforts. However, this leads to degradation in performance. Ex, due to reduced power consumption, steady state error may increase.

2]Rocket control or missile control system

High accuracy is needed, so we can not compromise on performance. Hence, Large Q values are selected. This penalizes low performance, but with increased fuel consumption.

- This is full state feedback control technique.

- Block diagram of the pole placement and the LQR controller is same, However, different methods used to calculate the feedback gain K in both the cases.

Problem Statement- To implement LQR control for DC motor and verify results by choosing different value of Q  and R.

Model

Parameters

(J)     moment of inertia of the rotor     0.01 kg.m^2

(b)     motor viscous friction constant    0.1 N.m.s

(Ke)    electromotive force constant       0.01 V/rad/sec

(Kt)    motor torque constant              0.01 N.m/Amp

(R)     electric resistance                1 Ohm

(L)     electric inductance                0.5 H

Matlab Code-

lqr(SYS,Q,R,N)close all;

% initialize state close all;

%

% initialize state

x0= [0.2;     % 200 mA

0.0872]   % rad/sec = 300 rpm

%motor dynamics

A = [-2 -0.02;

1   -0.01]

B = [2;

0]

C = [0 1]

D = [0]

Q = [1 0;

0 1]

R = [1]

%LQR command to find feeback gain K  ()

K = lqr(A,B,Q,R);

% close loop system

SYS= ss(A-B*K,B,C,D);

% creating time stamps

t= 0: 0.005 : 30;

% stimulate the system

[y,t,x] = initial(SYS,x0,t);

plot (t,y);

Comparison of graph for different choices of values of Q and R matrices.

Case 1 With same weightage given to both performance and actuator efforts

Case 2  Values of Q matrix are kept high while R = 1; system setles down quickly.

Q = [10 0

0 10]

Case 3  Q = diagonal matrix with all values equals to 1 and R = 10, penalizing actuator efforts.

setting time is increased.